## Alderbury Parish Council November2025

### Agenda item 122.25 c

### Alderbury Recreation Ground Inspection Findings

Shows items mentioned by professional inspectors as having a potential risk other than low, including where our inspectors found other issues

NB Graffiti is a problem on signage and the skatepark. Not a risk as such, but may be worth tackling.

Safety surface shrinkage throughout. Inspectors seem to vary on their assessment of this as a risk.

| EQUIPMENT | CONCERN | RISK – RoSPA October 2025 | RISK – Vita Play Aug 2025 | What our own inspections say (if different) | Further observations |
| --- | --- | --- | --- | --- | --- |
| Basket swing | Tape is not an appropriate solution to a known problem | medium | High |  | PC has agreed this is a risk they are prepared to live with |
| Toddler multiplay | Head entrapment | Medium |  |  | Playsafety admit nothing can be done |
|  | Platform is not secure | medium |  |  |  |
| Quarter pipe -1800mm | Ground to unit transition is too wide | Medium |  |  | Playsafety always say this and admit nothing can be done |
| Driveway (on skate park) | Edges not radiused or chamfered | Medium |  |  |  |
|  | Gap between plates too wide | Medium |  |  | No action recommended |
| Spine ramp and rollover | Edges not radiused or chamfered | Medium |  |  |  |